It’s Electric!

Hands-on Activity for Understanding Compasses and Electricity

Key Question

How can a compass be used as a galvanometer?

Background

A compass responds to magnetic fields.  Depending on the strength of the magnetic field, the direction of the compass needle can be misled.  An example of this could be if a compass is reading true north.  When the same compass is introduced to a strong magnetic field, it could read more northwest.

The Earth has a magnetic field.  A magnetic field can also be created when an electrical current passes through a wire.  The greater the current, the greater the magnetic field.  The stronger the magnetic field in relationship to the Earth’s magnetic field, the greater the effect on the compass.  A device called a galvanometer can measure electrical current that passes through a wire.  A compass wrapped with a thin wire can also measure an electrical current, therefore acting as a galvanometer.  Very small differences in electrical current can have a significant effect on a magnetic field produced by an electrical current.

Time—approximately 1 hour

30 minutes—setup

5-10 minutes—observations

15 minutes—concluding activity

Material and Preparation

- 3 small, 1 ½  X  ¼ inch (approximately) compasses

- 3—2 inch square pieces of cardboard

- 3—30 inch thin, enameled, copper wires

- electrical tape

- 1 AA-battery

- 1-D battery

Preparation

- With the compass needle pointing North/South, fold a small piece of cardboard ¼ inch on two sides and tape the two sides of the cardboard to the East/West sides of the compass.  (The cardboard makes it easier to wrap the thin wire around the compasses.)

- Strip ½ inch of enamel from both ends of each wire to permit easy contact with poles of AA and D cell batteries.

- Wrap a wire once in an East/West direction around the first compass.  Wrap the second compass with two turns of wire and the third with three turns.

Procedure

- Touch the leads from the compasses wrapped once with the wire and touch them to the positive and negative poles of the AA battery.

- Take the leads and touch them to the poles of the D cell battery.

- Record your observations.  Repeat steps 1 and 2 using the other two compasses.

Closure Questions

- What can you conclude from each compass and each battery?

- Determine which combination of compass and battery had the strongest magnetic field as measured by the movement of the compass needle.  Why did this happen?